# Givens

Zo=50Ω,  f=9.5GHz

Copper thickness=1 oz/ft2=34 μm

Substrate: Rogers RT/Duroid 5880

h=787.41 μm,  εr=2.20±0.04,  tanδ=0.0009 @ 10 GHz

npn type BJT BFG410W fabricated by NXP semiconductors

Ic=6mA,  Vce=2V

S parameters @9.5GHz are Extracted from Spar_BFG410W_commonemitter2V6mA file

S=0.608∠34.757°0.151∠-52.987°1.603∠-81.484°0.238∠49.648°

Matching Networks Microstrip TL — radial stub and λ/4 transmission line λ=λg=λoεeff

# Specs

- Stability: stable over the entire useful bandwidth of the device

- G=GT,max at the design frequency

- S11amp, S22amp: minimize

- Only open-circuited shunt stubs should be used

- BW: maximize, based on a narrowband design (The pair of networks that are to be used in the end should be selected such that the bandwidth of the amplifier is maximized)

- Amplifier must be DC-isolated by blocking capacitors

- Layout has space between the launcher microstrip lines and the surface mount capacitors

- Layout must take into consideration the physical dimensions of the transistor, capacitors, and resistors. The capacitors and resistors are surface-mounted ones. You can use the dimensions of the 0603 package for the capacitors and resistors.

# Stability

∆=0.366∠59.891°

∆=0.366<1

K=1.462>1

The device is unconditionally stable @9.5GHz

Another Solution

μ=1.849>1

The device is unconditionally stable @9.5GHz

# Gain

Law4

GT,max=4.199=6.231 dB

Another Solution

B1=1.179

B2=0.553

C1=0.53∠38.669°

C2=0.099∠118.652°

Γs=1.599∠-38.669° rejected,  0.625∠-38.669°

ΓL=5.404∠-118.652° rejected,  0.185∠-118.652°

Γin=0.625∠38.669°

Γout=0.185∠118.652°

Law1

GT=4.199=6.231dB

Law2

GT=4.199=6.231dB

Law3

GT=4.199=6.231dB

# Microstrip TL Design

A=1.159→Wd=3.126≮2 rejected

B=7.985→Wd=3.081>2

εeff=1.871

λ=λg=λoεeff=cofεeff=0.023069m

λ=23.069mm→λ4=5.767mm